| Parameter | Short Explanation | Unit | Detailed explanation | Python code line first mentioned | Imported from parameter sheet |
| --- | --- | --- | --- | --- | --- |
| Run_step_1 | Dummy to include hotplate step | - | Variable that includes the hotplate step if the value is 1, and excludes it in case it is 0 | 37 |  |
| Run_step_2 | Dummy to include preheating step | - | Variable that includes the preheating step if the value is 1, and excludes it in case it is 0 | 39 |  |
| HotplateSSTime | Duration of the hot plate turned on | s | The amount of seconds that the hot plate is on and is heated | 111 | ✔ |
| HotplateSSTimeInc | Time increment for hot plate heating calculations | s | Time increments in seconds, which controls solver time stepping for accuracy in hot plate heating calculations. Smaller increments give smoother temperature convergence | 112 | ✔ |
| HotplateTemp | Temperature of the hot plate when turned on | K | The temperature in Kelvin of the hot plate when it is on and heated | 113 | ✔ |
| film_ContactCoefHP | Film heat transfer coefficient for hotplate | μW/(mm²K) | Thermal contact conductance in μW/(mm²K) between hotplate and base, which determines rate of heat transfer at the interface | 114 | ✔ |
| FreeConvectionCf | Free convection coefficient | μW/(mm²K) | Coefficient for heat loss in μW/(mm²K) from exposed surfaces to ambient air via natural convection | 115 | ✔ |
| ambientTemp | Ambient temperature | K | Ambient temperature in Kelvin in the cold spray process room | 125 | ✔ |
| emissSubsDep | Emissivity of substrate-deposition surface | - | Emissivity of substrate-deposition surface which defines what percentage of black-body radiation the surface emits | 126 | ✔ |
| DepInitialTemp | Initial temperature of the deposited material | K | Temperature in Kelvin of the deposited material at the moment of deposit on the substrate | 128 |  |
| SubstrateTemp | Initial temperature of the substrate | K | Temperature in Kelvin of the substrate at the start of the cold spray process | 129 |  |
| BaseInitialTemp | Initial temperature of the base | K | Temperature in Kelvin of the base at the start of the cold spray process | 130 |  |
| baseWidth | Width of the base | mm | Width of the base in mm | 133 | ✔ |
| baseLength | Length of the base | mm | Length of the base in mm | 134 | ✔ |
| baseThick | Thickness of the base | mm | Thickness of the base in mm | 135 | ✔ |
| subsWidth | Width of the substrate | mm | Width of the substrate in mm | 138 | ✔ |
| subsLength | Length of the substrate | mm | Length of the substrate in mm | 139 | ✔ |
| subsThick | Thickness of the substrate | mm | Thickness of the substrate in mm | 140 | ✔ |
| depTrkWidth | Width of the deposition track | mm | Width of the deposition track in mm | 144 | ✔ |
| depTrkThick | Thickness of the deposition track | mm | Thickness of the deposition track in mm | 145 | ✔ |
| depTrksPerLyr | Number of deposition tracks per layer | - | Number of deposition tracks (side by side) that are required to deposit a layer | 146 | ✔ |
| depNoOfLyrs | Number of deposition layers | - | Number of deposited layers (on top of each other) in the cold spray process | 149 | ✔ |
| depWidth | Width of the deposited layer | mm | Width of the deposited layer in mm, so the widths of all deposition tracks in a layer together | 151 |  |
| depLength | Length of the deposited layer | mm | Length of the deposited layer in mm, so the length of a deposition track | 152 | ✔ |
| depThick | Thickness of all deposited layers combined | mm | Total thickness of all deposited layers (on top of each other) in mm | 153 |  |
| dep1Width | Width of the deposited material on the left side of the substrate | mm | Width of the deposited layer (all tracks together) in mm on the left side of the substrate | 158 |  |
| dep1Length | Length of the deposited material on the left side of the substrate | mm | Length of the deposited layer in mm on the left side of the substrate | 159 |  |
| dep1Thick | Thickness of the deposited material on the left side of the substrate | mm | Thickness of all deposited layers together in mm on the left side of the substrate | 160 |  |
| dep2Width | Width of the deposited material on the substrate | mm | Width of the deposited layer (all tracks together) in mm on the substrate | 162 |  |
| dep2Length | Length of the deposited material on the substrate | mm | Length of the deposited layer in mm on the substrate | 163 |  |
| dep2Thick | Thickness of the deposited material on the substrate | mm | Thickness of all deposited layers together in mm on the substrate | 164 |  |
| dep3Width | Width of the deposited material on the right side of the substrate | mm | Width of the deposited layer (all tracks together) in mm on the right side of the substrate | 166 |  |
| dep3Length | Length of the deposited material on the right side of the substrate | mm | Length of the deposited layer in mm on the right side of the substrate | 167 |  |
| dep3Thick | Thickness of the deposited material on the right side of the substrate | mm | Thickness of all deposited layers together in mm on the right side of the substrate | 168 |  |
| depVolThick | Thickness of a deposition volume (block) | mm | Thickness of one deposited volume in mm | 171 |  |
| depVolWidth | Width of a deposition volume (block) | mm | Width of one deposited volume in mm | 172 |  |
| depVolLength | Length of a deposition volume (block) | mm | Length of one deposited volume in mm | 173 |  |
| subsMatl | Material of the substrate | - | Material of which the substrate is made | 175 |  |
| depMatl | Deposited material | - | Material which is deposited on the substrate | 176 |  |
| BaseMatl | Material of the base | - | Material of which the base is made | 177 |  |
| PreHeatingTime | Duration of preheating step | s | Duration in seconds to preheat the part in the cold spray process | 180 | ✔ |
| PreHeatingTimeInc | Preheating time increment | s | Time increments in seconds, which controls solver time stepping for accuracy in preheating calculations | 181 | ✔ |
| DepositionTimeInc | Time increments for the deposition step | s | Time increments in seconds, which controls solver time stepping for accuracy in deposition calculations | 183 | ✔ |
| CoolingTime1 | Duration of the first clamped cooling step after deposition | s | Duration in seconds of the first clamped cooling step, where the part including deposited material is allowed to cool in ambient temperature while remaining clamped | 184 | ✔ |
| CoolingTimeInc1 | Time increments for the first clamped cooling step after deposition | s | Time increments in seconds for the first cooling step, which controls solver time stepping for accuracy in cooling calculations | 185 | ✔ |
| ReleaseFixturesTime | Duration of the step where the fixtures are released | s | Duration in seconds of the step where the fixtures are released. The boundary conditions applied before are deactivated in this step and the part is removed from the hotplate | 186 | ✔ |
| CoolingTime2 | Duration of the second cooling step after releasing the fixtures | s | Duration in seconds of the second cooling step, where the part including deposited material is allowed to cool in ambient temperature after the fixtures are released | 187 | ✔ |
| CoolingTimeInc2 | Time increments for the second cooling step after releasing the fixtures | s | Time increments in seconds for the second cooling step, which controls solver time stepping for accuracy in cooling calculations | 188 | ✔ |
| sprayNzzlVel | Spray nozzle velocity | mm/s | Spray nozzle velocity in mm/s, so the velocity at which the nozzle which deposits material moves over the substrate | 193 | ✔ |
| sprayNzzlAccel | Spray nozzle acceleration | mm/s2 | Spray nozzle acceleration in mm/s2, so the acceleration the nozzle uses to get to the required velocity | 194 | ✔ |
| sprayTimePerTrk | Time to deposit a single track | s | Time to deposit a single track in seconds, which consists of accelerating to the desired nozzle velocity and covering the deposition length | 196 |  |
| DepositionTime | Total deposition time | s | Total time to deposit the material in the cold spray process | 197 |  |
| TimeTotal | Total cold spray process time | s | Total time to complete the cold spray process, which includes heating the hot plate, preheating, depositing, cooling, releasing fixtures and cooling again | 211 |  |
| TimeStep01Begin | Time at which step 1 begins | s | Time at which step 1, heating the hot plate, begins | 213 |  |
| TimeStep01End | Time at which step 1 ends | s | Time at which step 1, heating the hot plate, ends | 214 |  |
| TimeStep02Begin | Time at which step 2 begins | s | Time at which step 2, preheating, begins | 216 |  |
| TimeStep02End | Time at which step 2 ends | s | Time at which step 2, preheating, ends | 217 |  |
| TimeStep03Begin | Time at which step 3 begins | s | Time at which step 3, depositing, begins | 219 |  |
| TimeStep03End | Time at which step 3 ends | s | Time at which step 3, depositing, ends | 220 |  |
| TimeStep04Begin | Time at which step 4 begins | s | Time at which step 4, clamped cooling, begins | 222 |  |
| TimeStep04End | Time at which step 4 ends | s | Time at which step 4, clamped cooling, ends | 223 |  |
| TimeStep05Begin | Time at which step 5 begins | s | Time at which step 5, releasing clamps, begins | 225 |  |
| TimeStep05End | Time at which step 5 ends | s | Time at which step 5, releasing clamps, ends | 226 |  |
| TimeStep06Begin | Time at which step 6 begins | s | Time at which step 6, free cooling, begins | 228 |  |
| TimeStep06End | Time at which step 6 ends | s | Time at which step 6, free cooling, ends | 229 |  |
| clampWidthXplus | Width of the clamp in the positive x-direction | mm | Width of the clamp in the positive x-direction in mm | 255 | ✔ |
| clampWidthXminus | Width of the clamp in the negative x-direction | mm | Width of the clamp in the negative x-direction in mm | 256 | ✔ |
| BiasMeshSubDir1Ratio | Bias mesh subdivision ratio in direction 1 | - | Scaling factor for biased mesh along direction 1, which ensures finer mesh in deposition region and coarser mesh elsewhere | 260 | ✔ |
| BiasMeshSubDir2Ratio | Bias mesh subdivision ratio in direction 2 | - | Scaling factor for biased mesh along direction 2 | 261 | ✔ |
| BiasMeshSubDir3Ratio | Bias mesh subdivision ratio in direction 3 | - | Scaling factor for biased mesh along direction 3, which ensures finer mesh in deposition region and coarser mesh elsewhere | 262 | ✔ |
| partition | Partition size for minimum mesh size in x-, y- and z-direction | - | Determines the partition of the minimum mesh size in the x-, y- and z-direction. Should always fit an integer amount in 1 | 275 |  |
| minMeshSizeX | Minimum element size in the x-direction | mm | Minimum element size in the x-direction, so element size in fine region | 277 |  |
| maxMeshSizeX | Maximum element size in the x-direction | mm | Maximum element size in the x-direction, so element size in coarse region | 278 |  |
| minMeshSizeY | Minimum element size in the y-direction | mm | Minimum element size in the y-direction, so element size in fine region | 280 |  |
| maxMeshSizeY | Maximum element size in the y-direction | mm | Maximum element size in the y-direction, so element size in coarse region | 281 |  |
| maxMeshSizeYBase | Maximum element size in the y-direction for the base region | mm | Maximum element size in the y-direction in the base region, so element size in coarse region | 282 |  |
| minMeshSizeZ | Minimum element size in the z-direction | mm | Minimum element size in the z-direction, so element size in fine region | 284 |  |
| minMeshSizeZBase | Minimum element size in the z-direction for the base region | mm | Minimum element size in the z-direction in the base region, so element size in fine region | 285 |  |
| maxMeshSizeZ | Maximum element size in the z-direction | mm | Maximum element size in the z-direction, so element size in coarse region | 286 |  |